L3L4CS Verification plan

Eden Shemesh

19.07.2023

This document contains Intellectual Property (IP) that is owned and belongs to GuardKnox Cyber Technologies Ltd. GuardKnox Cyber Technologies owns this document and its information and therefore it is forbidden to copy, duplicate, transfer and/or disclose any information, fully or partially, that is included in this document without written consent from GuardKnox Cyber Technologies Ltd.

REV 1.0

© GuardKnox Cyber Technologies Ltd.

Business Classification: Confidential

Changes History Table

| Rev. | Date | initiator | Change description | Approver |
| --- | --- | --- | --- | --- |
| 1 | 31/07/2023 | Eden Shemesh | Review 1 |  |

Content

1	Introduction	4

1.1	Scope and Objectives	4

1.2	Definitions, Acronyms & Abbreviations	4

1.3	Applicable Standard and Regulations	4

1.4	References	5

2	Dut Overview	6

2.1	Block diagram	6

2.2	Block Connection	6

2.3	Interfaces	7

2.4	Registers	8

3	Verification Plan	9

4	Verification Environment	10

4.1	Verification Environment Detailed Description	11

4.1.1	Verification Components Table	12

4.1.2	UVM Seq item	13

4.1.3	Simulation configuration	15

4.1.4	Compare testpoints	15

4.1.5	Interfaces vectors	15

4.1.6	Reference models	15

4.1.7	Data Flow and Operation	16

4.1.8	Pass/Fail criteria	16

5	Test Plan	17

6	Coverage	26

6.1	Code Coverage	26

6.2	Functional Coverage	26

Track functional coverage to ensure that the testcases cover different functional scenarios and configurations.	26

LIST OF FIGURES

Figure 1 – Top verification Environment diagram	7

Figure 2 – testbench diagram	8

Figure 3 – Reference model diagram	8

LIST OF TABLES

Table 1 – Definitions Table	4

Table 2 – Standard and Regulations Table	4

Table 2 – references documents Table	4

Table 4 – iterfaces Table	5

Table 5 – Registers Table	5

Table 6 – Verification Components Table	8

Table 6 – Testoints list Table	8

Table 7 – Test plan Table	10

Introduction

The document describes the verification goals for testing and the underlying requirements for generation, reference modelling and coverage.

## Scope and Objectives

This document describes the required functionality to be tested as well as required checkers, assertions. This document provides a detailed description of the verification environment(s) used to cover the verification of the design under test.

Definitions, Acronyms & Abbreviations

| Acronym | Definition |
| --- | --- |

Table 1 – Definitions Table

## Applicable Standard and Regulations

| Ref. | Doc Name | Link to ref doc |
| --- | --- | --- |
| [A1] | Automotive SPICE 3.1 VDA-HIS scope |  |
| [A2] | ISO 9001:2015 |  |
| [A3] | ISO 26262:2018 Functional Safety |  |

Table 2 – Standard and Regulations Table

## References

The following references contain useful information concerning the standard compatibility and other supportive documentation. The table contains links to all related documents, DUT specification, reference models specification, tests list spreadsheet, etc.

| Ref. | Doc Name | Author(s) | safety item | Link to ref doc |
| --- | --- | --- | --- | --- |
| [R1] | Checksum verification L3 & L4 | DOV MOSES | no | https://guardknox365.sharepoint.com/:p:/r/sites/hwteam/_layouts/15/Doc.aspx?sourcedoc=%7B4A02D120-6E98-431D-9009-B9CA1D402507%7D&file=L3%20%26%20L4%20Checksum%20Verfiction.pptx&action=edit&mobileredirect=true |
| [R2] | Detailed design document | DOV MOSES | no | https://guardknox365.sharepoint.com/:w:/r/sites/hwteam/_layouts/15/Doc.aspx?sourcedoc=%7BA879A003-E047-46E0-AD40-197A4971BF7E%7D&file=Detailed%20Design%20L4%20Checksum%20V1.5%20.docx&action=default&mobileredirect=true&cid=53450601-51bb-486c-b0a7-7c2beadf1570 |
| [R3] |  |  |  |  |
| [R4] |  |  |  |  |
| [R5] |  |  |  |  |
| [R6] |  |  |  |  |

Table 3 – references documents Table

Dut Overview

The L3L4CS block is designed to validate the checksum of L3 (Network Layer) and L4 (Transport Layer) in real-time. L3 and L4 Checksum outputs are valid when the ‘checksum valid’ signal is asserted.

This block works as part of the ‘Comm Engine’.

## Block diagram

Figure 1 – High Level Block Diagram

## Block Connection

Figure 2 – L3 L4 checksum block connections

## Interfaces

| AXI SLAVE |
| --- |
| idx | Name | Type | Dir | note |  |
| 1 | tvalid | bit | Input |  |  |
| 2 | tlast | bit | Input |  |  |
| 3 | tuser | bit | Input |  |  |
| 4 | tdata | bit [7:0] | Input |  |  |

Table 4 – iterfaces Table

| AXI MASTER |
| --- |
| idx | Name | Type | Dir | note |  |
| 1 | tvalid | bit | output | 4 clock delay |  |
| 2 | tlast | bit | output | 4 clock delay |  |
| 3 | tuser | bit [76:0] | output | 4 clock delay |  |
| 4 | tdata | bit [76:0] | output | 4 clock delay |  |

Table 5 – iterfaces Table

| Marker vector |
| --- |
| idx | Name | Type | Dir | note |  |
| 1 | Marker vector | Bit [29:0] | Input | This vector contains the various markers that are used Asserted when ‘1’. Not asserted when ‘0’. |  |

Table 6 – iterfaces Table

| CS outputs |
| --- |
| idx | Name | Type | Dir | note |  |
| 1 | Cs_valid_o | bit | output |  |  |
| 2 | L3_cs_o | bit [1:0] | output | Valid when cs_valid_o is ‘1’ |  |
| 3 | L4_cs_o | bit [1:0] | output | Valid when cs_valid_o is ‘1’ |  |
| 4 | length_error_o | bit | output | Error in ihl<5 or total_len<20 |  |
| 5 | L2_error_o | bit | output | Error in FCS |  |

Table 7 – iterfaces Table

| Clk&rst |
| --- |
| idx | Name | Type | Dir | note |  |
| 1 | Sys_clk | bit | input | The ststem clok |  |
| 2 | srst_n | bit | input | Synchronous reset_n |  |
| 3 | asrst_n | bit | input | ASynchronous reset_n |  |

Table 8 – iterfaces Table

| L3 CS output logic Table L3 CS output Default value 11 - no packet received. 11 – no packet was received. 10 – CS bad. 01 – CS good. 00 – non-IP. Table 9: L3 Output Logic Table L4 CS output logic Table L4 CS output Default value 11 - no packet received. 11 – no packet was received. 10 – CS bad. 01 – CS good. 00 – non-TCP/UDP. Table 10: L4 Output Logic Table |  |  |
| --- | --- | --- |

Registers

List of DUT’s and sub modules registers To level interfaces table & description

| idx | Register Name | Type | field | Description |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |
| 7 |  |  |  |  |
| 8 |  |  |  |  |
| 9 |  |  |  |  |

Table 11 – Registers Table

Verification Plan

The purpose of the verification plan is to outline the verification strategy and test methodology for the L3L4CS functionality in the DUT. the DUT is responsible for checking the is correctness in L3 and L4 eth packets in Real time. the verification environment consists one agent that generate various eth packets and drives them to the DUT via axis protocol. In the other side of the DUT there is another agent that called cs agent that collects and compare the cs result from the DUT.

The main objectives of this verification plan are as follows:

1.	Verify the correctness of the L3 and L4 checksum calculation in real-time.

2.	Ensure that the DUT properly detected and reports any checksum errors.

3.	Validate the DUT behavior under various traffic scenarios and stress condition.

4.	Achieve functional coverage goals to ensure comprehensive verification.

Verification environmant:

The verification environment comprises the following components:

- DUT (L3L4CS): The DUT checks the correctness of checksums in L3 and L4 Ethernet packets in real-time.

- Agent (Axi_wrapper_agent): This agent generates various Ethernet packets and drives them to the DUT using the AXI protocol.

- CS-Agent: The CS agent collects and compares the CS results from the DUT to validate the checksum calculation.

Verification methodolgy:

The verification methodology involves the following steps:

- Testplan Development: Develop a comprehensive testplan based on the DUT specification and functional requirements, including test scenarios and testcases.

- Functional Verification: Execute the testcases on the DUT using the testbench and verify the functionality according to the testplan.

- Code Coverage: Monitor and track code coverage metrics to ensure adequate testing of the DUT.

Verification Environment

Figure 3 – Top verification Environment diagram

Verification Environment Detailed Description

Figure 4 –verification Environment diagram

### Verification Components Table

verification components with detailed description

| idx | verification comp | Type | Description |
| --- | --- | --- | --- |
| 1 | Axi_wrapper_agent | Uvm_agent | Agent class itself is a top-level container class for the components and they are the bottom of test bench hierarchy. the components are eth_driver, eth_sequencer, eth_monitor. |
| 2 | Axi_wrapper _driver | Uvm_drvier | eth_driver Collects the eth frame data from the sequencer and sends to the dut via the axi interfacel. |
| 3 | Axi_wrapper monitor | Uvm_monitor | eth_monitor is a component which receives eth frame via axi inf from the DUT and and parse it and then send to the scoreboard via analysis port. |
| 4 | Eth_item_coverage | Uvm_subscriber | Eth_item_coverage collects eth_items for collection of a coverpoints. |
| 5 | CS agent | Uvm_agent | Agent class itself is a top-level container class for the components and they are the bottom of test bench hierarchy. the components in this agent is only cs monitor |
| 6 | CS_monitor | Uvm_monitor | cs_monitor is a component which receives cs outputs from the send it to the scoreboard via analysis port. |
| 7 | CS_item_coverage | Uvm_subscriber | Cs_coverage collects cs_items for a collection of a coverpoints. |
|  | Cs_Scoreboard | Uvm_scoreboard | Reciverd transcation of actual cs and compare with the transaction of the expected cs that generated in extract cs item inside axi wrapper agent. |
| 8 | test | Uvm_test | The test extends uvm_test. It consists of instances for sequences, environments and interfaces such as axi_stream_intf, further in the build phase object is created for components. In the run phase the basic sequence is created and started on the sequencer within phase. raise_objection and phase. drop_objection. |
| 9 | Env | Uvm_env | environment is a container component for assembling sub component of test like CS_agent and axi_wrapper_agent. |

Table 12 – Verification Components Table

### UVM Seq item

Figure 5 –eth_seq_item_frames

Figure 6 –cs_seq_item

### Simulation configuration

### Compare testpoints

| idx | Testpoint name | RTL loaction | Agent associate | Description |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |
| 6 |  |  |  |  |

Table 6 – Testoints list Table

### Interfaces vectors

### Reference models

Figure 3 – Reference model diagram

### Data Flow and Operation

The data flow in the L3L4CS verification environment follows these major steps:

1.Assert the “srst_n” signal to low for 10 us to reset the DUT.

2.Release the “srst_n” signal (set it high) to start the DUT operation.

3. The Axi_wrapper agent generates Ethernet packets (eth_item) according the test plan.

The axi_wrapper agent responsible drives the eth_item to the DUT using the AXI protocol.

4.the extract_cs_item component inside Axi_wrapper agents generate expected cs_item and pass it to the scoreboard via the analysis port inside cs agent.

5. the scoreboard inside the cs agent make the compression between the expected cs item to the actual cs item.

6. the scoreboard inside the axi_wrappermake the compression between the expected eth_item to the actual eth item.

### Pass/Fail criteria

The verification process will be considered complete and ready for signoff when:

- All Test Cases in the Testplan have been excuted and passed successfully.

- Code Coverage Goals have been met.

- Functional Coverage Goals have been achieved, covering the required Scenarios and configurations.

- Any reported Bugs have been resovled and verifed.

Test Plan

1. Packets without VLAN IPV4 UDP TEST:

Create random numbers of eth_itemes with diff size payloads (min 4 frames) with these fields:

- Eth_item. L2. with_vlan=0 //without vlan

- Eth_item. L2.ethertype=0800 // ipv4

- Eth_item. L4=8’h11 //udp

- Eth_item. L3_TOTAL_LEN [ 20: 65535]

In each packet the cs can be correct or wrong in L3 and L4.

Expected Result:

- cs_l3_o and cs_l4_o should be '01' or ‘10’, indicating correct CS or wrong CS.

- len_error_o and L2_error_o should be '0', indicating no length error or L2 error.

EX :

| Layer Number | Layer 2 |  |  |
| --- | --- | --- | --- |
| Title | MAC destination | MAC source | Ethertype |
| Number of Octets | 6 octets | 6 octets | 2 octets |
| Subtitle |  |  |  |
| Number of Octets |  |  | 0x0800 |

| Layer Number | Layer 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title | IPv4 Header |  |  |  |  |  |  |  |  |  |
| Number of Octets | 20 Octets |  |  |  |  |  |  |  |  |  |
| Subtitle | Version+IHL | DSCP+ECN | Total Length | Identification | Flags+ Offset | Time to Live | Protocol | Header Checksum | Source IP | Destination IP |
| Number of Octets | 1 Octet | 1 Octet | 2 Octets | 2 Octets | 2 Octets | 1 Octet | 1 Octet | 2 Octets | 4 Octets | 4 Octets |

| Layer Number | Layer 4 |  |  |  |  | Layer 2 |
| --- | --- | --- | --- | --- | --- | --- |
| Title | UDP Header |  |  |  | Payload | Frame check sequence (32bit CRC) |
| Number of Octets | 8 Octets |  |  |  | 1-65,508 Octets | 4 octets |
| Subtitle | Source Port | Destination Port | Length | Checksum |  |  |
| Number of Octets | 2 Octets | 2 Octets | 2 Octets | 2 Octets |  |  |

2. Packets with VLAN IPV4 UDP TEST:

Create random numbers of eth_itemes with diff size payloads (min 4 frames) with these fields:

- Eth_item. L2.with_vlan=1 //with vlan

- Eth_item. L2.ethertype=0800 // ipv4

- Eth_item. L4=8’h11 //udp

- Eth_item. L3_TOTAL_LEN [ 20: 65535]

In each packet the cs can be correct or wrong in L3 and L4.

Expected Result:

- cs_l3_o and cs_l4_o should be '01' or ‘10’, indicating correct CS or wrong CS.

- len_error_o and L2_error_o should be '0', indicating no length error or L2 error.

| Layer Number |  | Layer 2 |  |  |
| --- | --- | --- | --- | --- |
| Title | MAC destination | MAC source | VLAN | Ethertype |
| Number of Octets | 6 octets | 6 octets | 4 octets | 2 octets |
| Subtitle |  |  |  |  |
| Number of Octets |  |  |  |  |

| Layer Number | Layer 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title | IPv4 Header |  |  |  |  |  |  |  |  |  |
| Number of Octets | 20 Octets |  |  |  |  |  |  |  |  |  |
| Subtitle | Version+IHL | DSCP+ECN | Total Length | Identification | Flags+ Offset | Time to Live | Protocol | Header Checksum | Source IP | Destination IP |
| Number of Octets | 1 Octet | 1 Octet | 2 Octets | 2 Octets | 2 Octets | 1 Octet | 1 Octet | 2 Octets | 4 Octets | 4 Octets |

| Layer Number | Layer 4 |  |  |  |  | Layer 2 |
| --- | --- | --- | --- | --- | --- | --- |
| Title | UDP Header |  |  |  | Payload | Frame check sequence (32bit CRC) |
| Number of Octets | 8 Octets |  |  |  | 1-65,508 Octets | 4 octets |
| Subtitle | Source Port | Destination Port | Length | Checksum |  |  |
| Number of Octets | 2 Octets | 2 Octets | 2 Octets | 2 Octets |  |  |

3. Packets without VLAN IPV4 TCP TEST:

Create random numbers of eth_itemes with diff size payloads (min 4 frames) with these fields:

- Eth_item. L2. with_vlan=0 //without vlan

- Eth_item. L2.ethertype=0800 // ipv4

- Eth_item. L4=8’h06 //udp

- Eth_item. L3_TOTAL_LEN [ 20: 65535]

In each packet the cs can be correct or wrong in L3 and L4.

Expected Result:

- cs_l3_o and cs_l4_o should be '01' or ‘10’, indicating correct CS or wrong CS.

- len_error_o and L2_error_o should be '0', indicating no length error or L2 error.

Packet without VLAN IPV4 TCP:

| Layer Number | Layer 2 |  |  |
| --- | --- | --- | --- |
| Title | MAC destination | MAC source | Ethertype |
| Number of Octets | 6 octets | 6 octets | 2 octets |
| Subtitle |  |  |  |
| Number of Octets |  |  | 0x0800 |

| Layer Number | Layer 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title | IPv4 Header |  |  |  |  |  |  |  |  |  |
| Number of Octets | 20 Octets |  |  |  |  |  |  |  |  |  |
| Subtitle | Version+IHL | DSCP+ECN | Total Length | Identification | Flags+ Offset | Time to Live | Protocol | Header Checksum | Source IP | Destination IP |
| Number of Octets | 1 Octet | 1 Octet | 2 Octets | 2 Octets | 2 Octets | 1 Octet | 1 Octet | 2 Octets | 4 Octets | 4 Octets |

| Layer Number |  |  |  |  |  |  |  |  |  |  | Layer 4 |  |  |  |  |  |  |  | Layer2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title |  |  |  | TCP header |  |  |  |  |  |  |  |  |  |  |  |  | Payload |  | Frame check sequence (32bit CRC) |  |
| Number of Octets |  | 20 Octets |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1-65,508 Octets |  | 4 octets |  |
| Subtitle |  | Source_port |  |  | dest_port | Sequence number |  | Acknowledge number |  |  | Data offset | reserved | flags | window | checksum | Urgent pointer |  |  |  |  |
| Number of Octets |  | 2 Octet |  |  | 2 Octet | 4 Octets |  | 4 Octets |  |  | 0.5 Octets | 0.5 Octet | 1 Octet | 2 Octets | 2 Octets | 2 Octets |  |  |  |  |

4.Packets with VLAN IPV4 TCP TEST:

Create random numbers of eth_itemes with diff size payloads (min 4 frames) with these fields:

- Eth_item. L2.with_vlan=1 //with vlan

- Eth_item. L2.ethertype=0800 // ipv4

- Eth_item. L4=8’h06 //udp

- Eth_item. L3_TOTAL_LEN [ 20: 65535]

In each packet the cs can be correct or wrong in L3 and L4.

Expected Result:

- cs_l3_o and cs_l4_o should be '01' or ‘10’, indicating correct CS or wrong CS.

- len_error_o and L2_error_o should be '0', indicating no length error or L2 error.

| Layer Number |  | Layer 2 |  |  |
| --- | --- | --- | --- | --- |
| Title | MAC destination | MAC source | VLAN | Ethertype |
| Number of Octets | 6 octets | 6 octets | 4 octets | 2 octets |
| Subtitle |  |  |  |  |
| Number of Octets |  |  |  |  |

| Layer Number | Layer 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title | IPv4 Header |  |  |  |  |  |  |  |  |  |
| Number of Octets | 20 Octets |  |  |  |  |  |  |  |  |  |
| Subtitle | Version+IHL | DSCP+ECN | Total Length | Identification | Flags+ Offset | Time to Live | Protocol | Header Checksum | Source IP | Destination IP |
| Number of Octets | 1 Octet | 1 Octet | 2 Octets | 2 Octets | 2 Octets | 1 Octet | 1 Octet | 2 Octets | 4 Octets | 4 Octets |

| Layer Number |  |  |  |  |  |  |  |  |  |  | Layer 4 |  |  |  |  |  |  |  | Layer2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title |  |  |  | TCP header |  |  |  |  |  |  |  |  |  |  |  |  | Payload |  | Frame check sequence (32bit CRC) |  |
| Number of Octets |  | 20 Octets |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1-65,508 Octets |  | 4 octets |  |
| Subtitle |  | Source_port |  |  | dest_port | Sequence number |  | Acknowledge number |  |  | Data offset | reserved | flags | window | checksum | Urgent pointer |  |  |  |  |
| Number of Octets |  | 2 Octet |  |  | 2 Octet | 4 Octets |  | 4 Octets |  |  | 0.5 Octets | 0.5 Octet | 1 Octet | 2 Octets | 2 Octets | 2 Octets |  |  |  |  |

5. Length_error TEST:

Generate an Ethernet packet with valid L3 and L4 headers and their corresponding correct checksums. Introduce an error in the total length of the packet.

for ex: L3_total_len lower 20 or L3_ IHL<5.

Expected Result:

- len_error_o should be '1', indicating a length error.

- L2_error_o should be '0', indicating no L2 error.

6. invalid_L2_FCS TEST:

Generate an Ethernet packet with valid L3 and L4 headers and their corresponding correct checksums. Introduce an error in the L2 FCS after the L4 payload.

Expected Result:

- len_error_o should be '0', indicating no length error.

- L2_error_o should be '1', indicating an error in the L2 FCS after the L4 payload.

7. NO_Packet_received TEST:

Do not generate any Ethernet packet.

Expected Result:

- cs_l3_o and cs_l4_o should be '11', indicating that no packet was received.

- len_error_o and L2_error_o should be '0', indicating no length error or L2 error.

- Cs valid should be in zero.

8. Non-IP L3_Header TEST:

Generate an Ethernet packet with a non-IP L3 header (e.g., ARP, PTP) and with correct cs in L4.

Expected Result:

- cs_l3_o should be '00', indicating a non-IP L3 header.

- cs_l4_o should be '00', indicating a correct checksum in L4.

- len_error_o and L2_error_o should be '0', indicating no length error or L2 error.

9. Non-TCP/UDP L4_Header TEST:

Generate an Ethernet packet with a non-TCP/UDP L4 header (e.g.,) and with correct cs in L3.

Expected Result:

- cs_l3_o should be '01', indicating a correct checksum in L3.

- cs_l4_o should be '00', indicating a non-TCP/UDP L4 header.

- len_error_o and L2_error_o should be '0', indicating no length error or L2 error.

RANDOM TESTS:

1. Continuous_Packet_Stream TEST:

Generate a continuous stream of Ethernet packets with different combinations of L3 and L4 headers and min supported rate.

Expected Result:

- Verify that the DUT can handle the continuous stream without errors and report any issues correctly.

2. random_payload_len TEST:

Generate Ethernet packets with random payload lengths (within the valid range) and correct checksums in L3 and L4 headers.

Expected Result:

- Verify that the DUT can handle packets with various payload lengths without errors.

CORNER TESTS:

1. Boundary values TEST:

Generate Ethernet packets with checksum values at the maximum and minimum allowable range.

Expected Result:

- Ensure that the DUT handles boundary values and detects errors.

2. handling Malformed Packets TEST:

Verify the DUT's response when processing malformed packets with invalid or corrupted headers. Generate Ethernet packets with invalid or corrupted L3 and L4 headers.

Expected Result:

- Verify that the DUT handles malformed packets without any assertion or critical errors. It should produce appropriate output values for cs_l3_o, cs_l4_o, len_error_o, and L2_error_o.

3. Invalid_len_& L2 & Incorrect CS Packets TEST:

Verify the DUT's response when the packet length is incorrect, L2 is incorrect, and there is also an incorrect checksum in L3, L4.

Generate an Ethernet packet with an incorrect total length and incorrect checksums in both L3 and L4 headers.

Expected Result:

- cs_l3_o and cs_l4_o should be '10', indicating incorrect checksums in both headers.

- len_error_o and L2_error_o should be '1', indicating length error and L2 error.

4. Jumbo frame TEST:

Generate Ethernet packets with max payload length for creating jumbo frame with different combinations of L3 and L4 CS fileds.

Expected Result:

- Ensure that the DUT handles the frames and detects cs results.

5. srst_during_frame TEST:

Generate Ethernet packets and toggling the srst during the hdr transmission of in the transmission.

Expected Result:

Ensure that the DUT detects srst and react correctly.

Coverage

Code Coverage

Functional Coverage

- Track functional coverage to ensure that the testcases cover different functional scenarios and configurations.

| cover group | Cover Point | Width | cover bins | status |
| --- | --- | --- | --- | --- |
| Cvg_cs_outputs | Cvr_l3_cs | 2 bits | l3_cs_o= {01,10,00,11}; |  |
| Cvg_cs_outputs | Cvr_l4_cs | 2 bits | L4_cs_o= {01,10,00,11}; |  |
| Cvg_cs_outputs | Cvr_len_er_o | bit | len_er_o {0,1}; |  |
| Cvg_cs_outputs | Cvr_L2_er_o | bit | L2_er_o {0,1}; |  |
| Cvg_eth | Cvr_eth_L3_length | 2 bytes | L3_length |  |
| Cvg_eth | Cvr_eth_L3_IHL | 4bits | L3_IHL {[0:5} [5:10] |  |
| Cvg_eth | Cvr_eth_L3_protocol | byte | L3_protocol {8’h11,8’h06} {arp, ptp,0…} |  |
| Cvg_eth | Cvr_eth_L2_FCS | 4 bytes | L2.FCS= |  |
| Cvg_eth | Cvr_eth_L4_length | 2 bytes | L4.length=] |  |